FFVP

(Fédération Française de Vol en Planeur)

SLM Certification Programme

# Amendment Record

| Issue | Amendment Description | Date |
| --- | --- | --- |
| 01 | Initial Issue + update following experts review | 07/08/2026 |
| 02 | Update following continued review and consistency with MR | 25/08/2026 |

# Table of Content

0.1	Amendment Record	1

0.2	Table of Content	2

1.	Introduction	3

1.1	Project Description	3

1.2	Project History	4

1.3	Design Description	4

1.4	Design Organisation Resources	4

1.5	Design Subcontractors	5

1.6	Technologies and Process Description	5

2.	Type Certification Basis and Environmental Protection Requirements	6

2.1	Type Certification Basis	6

2.2	Equivalent Safety Findings (ESF), Deviations	6

2.3	Special Conditions	7

2.4	Environmental Protection	7

2.5	Operation Suitability Data (OSD) – Master Minimum Equipment List (MMEL)	7

3.	Certification Process	7

3.1	Means of Compliance	8

3.2	Certification Schedule	9

3.3	Approval of Flight Conditions	9

3.4	Engine/Propeller Certification and Interactions with Aircraft	9

3.5	Equipment Qualification	9

3.6	Control of Production and Configuration	10

3.7	Compliance Documentation	10

3.8	Type Design Definition	10

3.9	Master Document List	11

3.10	Final Declaration of Compliance	11

3.11	Proposed EASA Involvement	11

4.	List of Annexes	13

ANNEX A: CONOPS	14

CONOPS can be divided in 3 main phases:	14

Phase 1 — Instrumentation and Data Collection (below the approved Service Life Limit)	14

Phase 2 — EFI Evaluation when reaching the approved Service Life Limit	14

Phase 3 — Operation Beyond the approved Service Life Limit	15

ANNEX B: Master Document List	16

ANNEX C: Compliance Check List	17

# Introduction

This Certification Programme (CP) defines the compliance approach for an EASA Supplemental Type Certificate (STC) allowing extension of the operational life limit of selected gliders (Schempp-Hirth and LTB Lindner) using a Structural Life Monitoring (SLM) system. By tracking the Equivalent Fatigue Index (EFI) of each individual glider, the SLM system may support operation beyond the approved Service Life Limit, provided the aircraft remains within the approved SLM applicability domain and complies with the approved CONOPS, the Aircraft Flight Manual Supplement / Aircraft Maintenance Manual Supplement (AFMS-AMMS) — which provides the Instructions for Continued Airworthiness (ICA) content — required inspections, and EFI go/no-go criteria.

Some glider models included in the STC scope may have been originally certified by national European aviation authorities or under JAA arrangements before EASA assumed responsibility for the product. For the purpose of this STC, only aircraft types with an EASA-recognized Type Certificate Data Sheet and listed in the EASA product list are considered.

The Certification Programme will be updated, as required, to reflect any relevant changes to the project.

## Project Description

The STC introduces a system-based life-extension method using an accelerometer-based recorder installed in the fuselage, in a rigid area close to the C.G. and wing spar, and an analytical EFI calculation. No structural modification is required. The intent of this STC is not to change the structural fatigue requirements or limits defined in the original Type Certificate (TC), or to reopen or reinterpret the original structural substantiation. SLM calculates the cumulative EFI of an individual aircraft and compares it with the reference EFI associated with the approved Service Life Limit, using the Kossira & Reinke Utility reference spectrum. The reference EFI at the approved Service Life Limit is normalized to EFI = 1. The method does not claim to measure the absolute physical damage state of the composite structure.

The following items shall be approved within the STC:
- SLM methodology (principles, assumptions, limitations, EFI calculation method and selected reference spectrum)
- SLM hardware installation (as minor change via CS-STAN)
- SLM operational and maintenance procedures (Concept of Operations / CONOPS), including continued-airworthiness processes linked to EFI status and decision criteria for operation while the cumulative EFI remains below the SLM reference EFI limit (EFI = 1)

## Project History

Glider types involved in the experimentation, under the STC, have accumulated extensive operational experience. The STC builds upon existing fatigue-life knowledge and introduces monitoring-based operation while the cumulative EFI remains below the SLM reference EFI limit rather than structural margin re-evaluation or structural redesign.

## Design Description

The SLM modification adds a lightweight accelerometer-based recorder, post-flight analysis tools, and an updated AFMS-AMMS where applicable. No aerodynamic or structural modifications are required, and the certified aircraft flight envelope is not changed. However, SLM credit is limited to operation within the approved SLM applicability domain, namely Utility-category / non-aerobatic operation as defined in the SLM Methodology Report, CRI, and the AFMS-AMMS.

Hardware: An SLM recorder installed in the fuselage, in a rigid area close to the C.G. and wing spar, to measure in-flight accelerations used for EFI calculation.

Software/Process: A data analysis toolset for post-flight EFI evaluation based on the Palmgren-Miner rule, the Kossira & Reinke counting method and the associated Utility reference spectrum.

Key Advantage: this approach does not require reopening original Type Certificate (TC) structural justification documents, as it relies on the reference EFI associated with the approved Service Life Limit, normalized to EFI = 1. The applicable Service Life Limit is currently 12,000 hours for the gliders in this STC's scope per their TCDS.

## Design Organisation Resources

FFVP DOA provides engineering, airworthiness and programme management resources. Flight dynamics as well as structure and maintenance experts support the development of the SLM methodology.

## Design Subcontractors

Two companies are going to support this programme:
- Schempp-Hirth Flugzeugbau GmbH
- LTB Lindner
They are the TC holders for the gliders part of this STC.
They will review the STC documents and contribute with their expertise, to the establishment of recurring test covering both composite and non-composite parts. Since they are the TC holders and have access to the complete STC data package, they will be free to develop modifications and associated Service Bulletins to distribute SLM more broadly.

## Technologies and Process Description

Technologies include lightweight sensing, digital data recording, analytical EFI modelling, and associated data processing. Processes include calibration, data validation, cumulative EFI tracking and remaining EFI potential calculation, providing the opportunity for operation beyond the approved Service Life Limit while the cumulative and projected EFI remain below the SLM reference EFI limit.

Although innovative in the civil glider context, the SLM solution relies on estimation methods (e.g. Palmgren-Miner rule) that are well established and accepted across the aerospace industry. Similar measured-usage principles have been used in aviation usage monitoring and Individual Aircraft Tracking, particularly in military aviation. The SLM proposal does not attempt to reproduce those military approaches directly or claim that they are directly transferable to gliders. The relevant precedent is the general principle that measured usage can be compared with a conservative reference usage assumption, provided the method is controlled, conservative, and supported by inspections. SLM leverages the significant work of Kossira & Reinke, who established filtering, cycle-counting methods and used Markov transition matrices to develop glider load spectra officially referenced by EASA in Certification Memorandum CM-S-006.

# Type Certification Basis and Environmental Protection Requirements

The STC certification basis follows CS‑22, with primary focus on CS 22.627 Fatigue. Even if no environmental requirements apply to this STC, it contributes to lower the environmental impact by increasing life usage of composite gliders, for which no recycling solutions are available.

## Type Certification Basis

Applicable requirements: CS-22, Part 21 Subpart E. Primary requirement: CS 22.627 Fatigue.
Since there are no Interpretative Material (IM) or Acceptable Means of Compliance (AMC) to support the proposed SLM solution, a CRI will have to be issued to provide the required Means of Compliance material for the SLM fatigue-monitoring method.
The CRI shall cover acceptable:
- EFI scope and calculation method based on the Palmgren-Miner rule
- Kossira & Reinke Utility reference spectrum
- conservative composite severity parameter
- onboard recording solution and installation/recurring calibrations
- post-processing solution and tool validation
- treatment of invalid, missing and non-flight data
- CONOPS and Instructions for Continued Airworthiness (including impact on the AFMS-AMMS where applicable)
- limits of proposed methodology and SLM applicability, including exclusion of aerobatic and non-standard operations
- go/no-go criteria based on cumulative EFI total < 1 and projected EFI before the next approved review point < 1

This CRI should provide means of compliance allowing a glider equipped with SLM to operate beyond the approved Service Life Limit, provided that the cumulative EFI remains below the SLM reference EFI limit and the approved inspection and continued-airworthiness provisions are satisfied.

## Equivalent Safety Findings (ESF), Deviations

None anticipated. No deviations from CS‑22 are required.
 - The STC stays within CS-22 intent
 - Equivalence is shown analytically and operationally
 - Any ESF discussion, if needed, will be embedded in the CRI

## Special Conditions

None expected.

## Environmental Protection

Not applicable to this STC.

## Operation Suitability Data (OSD) – Minimum Equipment List (MEL)

No OSD items are required since there is no change to the operation of the glider within the approved flight envelope. MEL is not affected before the approved Service Life Limit is reached.
The SLM recorder is optional prior to reaching the approved Service Life Limit (currently 12,000 hours for the gliders in this STC's scope). However, to benefit from SLM, the recorder needs to be operated before the Service Life Limit so that valid measured-usage data can be credited.
Once operation exceeds the approved Service Life Limit, the SLM system becomes mandatory, and its availability, calibration status, validated EFI reports and associated inspections become conditions for continued airworthiness.

# Certification Process

## Means of Compliance

MoC shall be the result of the proposed CRI.

- EFI calculation methods: reference-versus-measured EFI comparison using the Palmgren-Miner rule as a common comparison framework

- reference load spectrum and composite severity parameter: normalization of the EFI calculation to the reference EFI associated with the approved Service Life Limit, using the Kossira & Reinke Utility spectrum and the conservative composite parameter defined in the SLM Methodology Report. This is required to calculate cumulative EFI for measured operation and reference-rate substitution

- onboard recording solution: sensor installation and calibration procedures

- post processing solution: toolset validation method

- CONOPS and Instructions for Continued Airworthiness: AFMS-AMMS integration where applicable, mandatory inspections, EFI status reports, and Go / No-Go criteria

- limits of SLM applicability/methodology (e.g. approved operating domain, exclusion of aerobatic or non-standard operations, and substitution procedure when SLM data is inaccurate or unavailable)

No flight testing is required since data will be gathered transparently during normal operations.

The Instructions for Continued Airworthiness (ICA) content required to support SLM shall be prepared and provided through the AFMS-AMMS, in accordance with Certification Specification CS-22.1529.

It should be understood that SLM is mandatory for operation beyond the approved Service Life Limit and that sensor calibrations, validated EFI status reports and inspections are release-to-service conditions.

While the glider is operated before the approved Service Life Limit, no measured-usage credit can be granted for operational time not supported by valid aerodynamic SLM measurements. This includes aircraft time accumulated before valid SLM monitoring begins, excluded non-aerodynamic phases, and missing or invalid data sequences. All such time is assigned EFI at the reference rate in accordance with the approved data-processing procedure.

Abnormal and failure cases shall be addressed within the AFMS-AMMS and CRI, including loss of recorded data, missed calibration, sensor malfunction, or data inconsistency. In such cases, continued operation beyond the approved Service Life Limit is not permitted unless corrective actions are taken and compliance with the approved SLM process is restored.

## Certification Schedule

The schedule should be provided following the phases described above and should include the major milestones, for example:

T0: CP Submission.

T0 + 3 Months: CRI Acceptance.

T0 + 12 Months: Full compliance documentation submitted.

T0 + 18 Months: Anticipated STC issuance.

## Approval of Flight Conditions

No special flight conditions required. No certification flight testing.

## Engine/Propeller Certification and Interactions with Aircraft

Not applicable; SLM does not affect propulsion.

## Equipment Qualification

Recorder is qualified and validated through analytical justification using MEMS sensor manufacturer specifications as well as functional tests, calibration checks, and data-integrity verification. This includes a calibration with 6-face rotation using gravity as a reference as well as an installation calibration with the glider positioned wings level and in the attitude specified for Centre-of-Gravity (CG) determination in the aircraft Maintenance Manual.

## Control of Production and Configuration

Recorder installation configuration is tracked using Certification Specifications for Standard Changes and Standard Repairs (CS-STAN) process. Each aircraft installation is traceable.

- One recorder per aircraft
- Aircraft-specific serialisation of data (data specific to glider)
- Central repository as a controlled source of truth
- Protection against selective data deletion or filtering (aircraft logbook is the reference)

The central SLM data repository constitutes the authoritative source for EFI data used for airworthiness decisions. EFI status reports used for ARC and continued airworthiness shall be generated and validated in accordance with the approved SLM procedures.

## Compliance Documentation

Compliance includes:
- SLM Certification Programme
- Certification Review Item or equivalent means-of-compliance agreement
- SLM Methodology Report
- Aircraft Flight Manual Supplement / Aircraft Maintenance Manual Supplement (AFMS-AMMS)
- SLM data-processing tool validation report
- Master Compliance Report

## Type Design Definition

The SLM programme does not modify the structural or aerodynamic configuration of the aircraft, the certified flight envelope, or the structural limits defined in the Type Certificate. The SLM recorder is installed in accordance with the applicable approved installation basis.
The approved change introduces an individual-aircraft monitoring-based means to compare cumulative operational EFI with the SLM reference EFI limit associated with the approved Service Life Limit.
- No change to certified operating limitations.
- No new mandatory equipment within the originally certified life limit.
- SLM equipment, calibration, validated EFI reports and inspections become conditions for continued airworthiness when operating beyond the approved Service Life Limit.

## Master Document List

An MDL will be issued listing CP, CRI, SLM Methodology Report, AFMS-AMMS, validation reports, data repository / data integrity procedure, approved calculation package and compliance data.

## Final Declaration of Compliance

Issued upon completion and EASA acceptance of compliance data / reports.

## Proposed EASA Involvement

| Compliance Data / Activity | Proposed EASA Involvement | Rationale |
| --- | --- | --- |
| CRI (SLM Means of Compliance) | Approval | To validate the overall SLM concept and operation while the cumulative EFI remains below the SLM reference EFI limit. Agree on the MoC statement. |
| SLM Methodology Report | Review | To agree with the EFI calculation methodology and normalization of the reference EFI associated with the approved Service Life Limit, including agreement on the selected Utility reference spectrum and composite severity parameter. |
| AFMS-AMMS | Review | To agree on compliance with Part ML and continued-airworthiness provisions, including inspections, release-to-service conditions, and EFI status reporting. This includes recorder and installation calibrations, recorder operation, maintenance and data handling. |
| Tools Validation | Review | To agree with the solution to validate tools for EFI calculations. |
| Master Compliance Report | Approval | Final sign-off on the demonstration of compliance for the STC. |

# List of Annexes

Annex A:  CONOPS

Annex B:  Master Document List

Annex C:  Compliance Check List

## ANNEX A: CONOPS

## CONOPS can be divided in 3 main phases:

### Phase 1 — Instrumentation and Data Collection (below the approved Service Life Limit)

This phase covers the period during which the SLM recorder is installed and the glider has not yet reached its approved Service Life Limit. It includes:

- Sensor installation in accordance with CS-STAN / Form 123

- Sensor installed in the fuselage, in a rigid area close to the C.G. and wing spar, and calibrated with the glider positioned wings level and in the attitude specified for Centre-of-Gravity (CG) determination in the aircraft Maintenance Manual.

- Annual 6-face calibration during the annual inspection

- Compliance with Part ML; installation and annual calibration results recorded in the aircraft logbook

- Reference-rate substitution: all operational time not supported by valid aerodynamic SLM measurements is assigned EFI at the Kossira & Reinke Utility reference rate and receives no measured-usage credit. This includes aircraft time accumulated before valid SLM monitoring begins, excluded non-aerodynamic phases, and missing or invalid data sequences, as defined in the approved ICA and data-processing procedure

- Flight data uploaded to the controlled central SLM repository; post-processing tools providing EFI status reports

### Phase 2 — EFI Evaluation when reaching the approved Service Life Limit

This phase includes:

- a fatigue inspection of the aircraft condition (corresponding to the existing 1,000-hour inspections).

- an assessment of EFI data, to ensure EFI quality for all the periods contributing to the EFI calculation and determine whether:
. cumulative EFI total < 1;
. projected EFI at the next scheduled inspection or approved review point remains < 1

- inspection beyond the composite structure, with an overall assessment of glider’s condition (aircraft logbook, traceability of incidents/repairs).

### Phase 3 — Operation Beyond the approved Service Life Limit

This phase is a continuation of Phase 1 monitoring operations, with the following additional provisions:

- Go / No-Go decision at each annual visit, Airworthiness Review Certificate (ARC) renewal, or other approved review point, based on the validated SLM EFI status report

- Go criterion: cumulative EFI total < 1, AND projected EFI at the next scheduled inspection or approved review point < 1

- Recurring structural inspection continuing from Phase 2, with a maximum interval of 1,000 hours or 5 years, whichever is sooner.

- This process continues until the aircraft has exhausted its remaining EFI potential, another approved continued-airworthiness limit is reached, or the aircraft is retired from service

## ANNEX B: Master Document List

| Reference | Title | Issue | Date |
| --- | --- | --- | --- |
| SLM-CP | Certification Programme | 01 | 05/07/2026 |
| SLM-CRI | Certification Review Item | 01 |  |
| SLM-MR | Methodology Report | Ed. 1 Rev. 2 | 23/08/2026 |
| SLM-AFMS-AMMS | Aircraft Flight Manual Supplement / Aircraft Maintenance Manual Supplement | 01 |  |
| SLM-TOOL | SLM tool validation report | 01 |  |
| SLM-MCR | Master Compliance Report | 01 |  |

## ANNEX C: Compliance Check List

| Req | Requirement Summary | MoC | Compliance Document | EASA Involvement | Status |
| --- | --- | --- | --- | --- | --- |
| CS 22.627 | Alternative MoC for SLM EFI-based fatigue life management | CRI | SLM-CRI, SLM-MR | Approval | Open |
| CS 22.1529 | Instructions for Continued Airworthiness, including inspections, release-to-service conditions and EFI status reporting | Doc. Review | SLM-AFMS-AMMS | Review | Open |
| CS 22.1585 | Operating procedures for SLM recorder, installation calibration, annual 6-face calibration and data upload | Doc. Review | SLM-AFMS-AMMS | Review | Open |
| Part ML | Continued airworthiness, Part ML process and ARC support using validated EFI status reports | Doc. Review | SLM-AFMS-AMMS | Review | Open |