| CERTIFICATION REVIEW ITEM |  |  |  |  |
| --- | --- | --- | --- | --- |
| APPLICANT: PROJECT NO.: PROJ. TITLE: APPLICABILITY: | FFVP |  | CRI: ISSUE: DATE: STATUS: Next ACTION: | 01 13/05/2026 Draft xxxxx |
| SUBJECT: |  | Structural Life Monitoring |  |  |
| CATEGORY: |  | Means of Compliance (MoC) |  |  |
| REQUIREMENTS: |  | CS 22.627 CS 22.1529 CS 22.1585 |  |  |
| ADVISORY MATERIAL: |  | EASA CM-S-006 Issue 01 |  |  |
| PRIMARY PANEL: |  | 03 |  |  |
| SECONDARY PANEL: |  | 05 |  |  |
| PUBLIC CONSULTATION |  | Not applicable |  |  |

IDENTIFICATION OF ISSUE

CS-22 currently recognizes several Acceptable Means of Compliance to demonstrate fatigue strength under CS 22.627, including a full-scale fatigue test based on an accepted reference load spectrum (e.g. Kossira & Reinke spectrum) or a static overload test at ultimate load plus additional safety margin.

In all cases, EASA grants a safe-life limit based on a conservative flight mixture utilization, which should cover all possible type operations.
This limit was originally established by the LBA (Luftfahrt-Bundesamt). The significant margin reflects the limited understanding of composite fatigue behavior at that time. However, decades of operational experience, aerospace-industry composite data, full-scale fatigue tests, and validated analytical comparisons, confirm that the 12,000-hour limit is conservative with respect to measured operational usage.
This conservatism is not only due to the limited historical knowledge of composite fatigue behavior, but also to the fact glider utilization (reference spectrum) provides a full operational usage envelope, which may exceed operational usage for a significant portion of the fleet.

Individual Aircraft Tracking (IAT), based on measured operational loads and cumulative fatigue accounting, has been widely used in military aviation for fatigue management of aircraft subject to variable operational usage.
IAT, using onboard acceleration measurement, Miner-Palmgren damage accumulation, and comparison against a reference spectrum, is standard practice in military aviation and operationally validated on numerous aircraft types over more than 50 years.

Although innovative in the civil glider context, the proposed Structural Life Monitoring solution (SLM) applies the same fundamental principles as IAT, adapted to the glider context.

SLM offers an alternative path, by instrumenting gliders with accelerometers and applying the Miner-Palmgren fatigue damage rule to real flight data. It can compare the actual damage accumulation to the reference damage accumulation on which the safe-life limit is based. If the measured cumulative damage remains below the certified fatigue damage limit, continued operation beyond the certified safe-life limit may be permitted.

This CRI establishes the Means of Compliance for SLM as an alternative fatigue-life justification method under CS 22.627, consistent with EASA CM-S-006 and without requiring any change to the certified fatigue damage limit or the TC structural documentation.
The proposed SLM approach does not establish a new structural fatigue life limit, but provides an alternative means to determine when the originally certified fatigue damage limit is reached on an individual-aircraft basis.

DISCUSSION

EASA POSITION (Issue 1, dated xx-xxx-yyyy) (drafted by applicant)
EASA may accept Structural Life Monitoring as an Alternative Means of Compliance to CS 22.627 for individual composite sailplanes certificated under CS-22 with an established safe-life limit, provided all requirements of Appendix A are met. This acceptance: (a) applies on a per-aircraft serial number basis; (b) does not modify the certified fatigue damage limit; (c) is limited to operations within the originally certified flight envelope; and (d) is conditioned on continuous SLM data validity — any period without valid recorded data shall revert to reference-spectrum fatigue accounting.

APPLICANT’S POSITION (Issue xx, dated xx-MAY-2026)

The FFVP agrees with the EASA position and will use the MoC for the compliance demonstration.

CONCLUSION (Issue xx, date)

TBD

CRI REVISION RECORD

| Issue | Date | CRI Changes |
| --- | --- | --- |
| 1 |  | Initial issue |

### Appendix A – Applicant Proposed SLM Compliance Methodology MOC XXX.XXX-XX - Means of Compliance for Structural Life Monitoring

The following MoC is proposed for Structural Life Monitoring:

For a glider with a certified fatigue damage limit and associated safe-life limit, a system for monitoring in-service fatigue usage may permit operation beyond the originally established safe-life limit, provided that the cumulative in-service fatigue does not exceed the certified fatigue damage limit.
The SLM approach does not modify the certified flight envelope, certified structural loads, or original structural substantiation.
The SLM methodology, associated parameters, processing tools, and configuration assumptions shall be subject to configuration control under the applicable design approval holder procedures.

Definitions:
Certified fatigue damage limit:  corresponds to the cumulative fatigue damage associated with the originally certified safe-life limit.

Safe-life limit:  corresponds to the approved operational life limit established during certification.

Reference spectrum: corresponds to the accepted load spectrum used for fatigue substantiation.

- Damage accumulation calculation method:
Data acquisition, filtering and spectrum calculation should be consistent with the selected reference spectrum accepted by EASA. 
The counting methodology shall be consistent with the reference spectrum accepted for fatigue substantiation (e.g. Kossira & Reinke as described in IFL-IB 84-01).
The damage calculation will use an industry-recognized methodology (e.g. Miner rule damage = sum( ni/Ni ) with damage = 1 corresponding to the certified fatigue damage limit).
The material stress-versus-fatigue-life model for high-cycle fatigue shall use an industry-recognized fatigue analysis methodology (e.g. Basquin formulation).
Where uncertainty exists, conservative assumptions shall be used for fatigue accounting.

- Reference load spectrum, composite material parameter and safety margin:
A load spectrum recognized by EASA shall be used as a reference load spectrum (e.g. Kossira & Reinke load spectrum from IFL-IB 84-01)
Conservative material fatigue parameters and safety assumptions representative of composite sailplane structures shall be used (e.g. when using Basquin rule, conservative material fatigue parameters covering the envelope of composite material used in the applications, as well as a conservative safety factor, both consistent with recognized aerospace fatigue-analysis practice).

- Installation of the SLM onboard recorder:
A recorder shall be installed at a representative structural location (e.g. wing spar on center line). The installation approval basis for the SLM recording system shall be established under the applicable change approval process (e.g. CS-STAN).
The recorder installation shall not alter structural load paths or flight characteristics.
The required documentation should be updated:  Aircraft Flight Manual (AFM), Aircraft Maintenance Manual (AMM) and Aircraft Maintenance Program (AMP).
The recorder shall be capable of generating digital files with g-load on 3 axes and its performance shall be validated (e.g. using scheduled calibration).

- Post processing toolset:
The authoritative fatigue status shall be generated from controlled and traceable data sources.
A centralized database shall collect, verify integrity, provide traceability for flight digital files and logbook information for each monitored glider.
An associated toolset shall process the digital files and provide damage calculation, remaining damage potential and remaining operational life, following the method described in 1.
The toolset performance shall be validated (e.g. reference data sets could be used to verify toolset performance).

- Concept of Operation and Instructions for Continued Airworthiness (ICA):
Operation beyond the certified safe-life shall require valid fatigue status determination.
Responsibilities associated with data acquisition, fatigue-status processing, continued airworthiness assessment, and operational release shall be defined within the applicable ICA.
AFM shall provide a description of the digital recorder operation.
AMM shall describe the required maintenance actions (e.g. recorder calibration during annual inspection).
AMP shall be updated to provide:
- annual inspection and requirement to file fatigue data (e.g. remaining damage potential, corresponding remaining operational life and determination of continued eligibility for Aircraft Release to Service (ARS)).
- specific inspection at initial safe-life limit (e.g. inspection at 12000-hour to verify overall glider situation, major repair history, aging structural inspection and remaining damage potential, to authorize continued operation beyond the initial safe-life limit and eligibility of ARS).
- repetitive (e.g. 1000 hours / 5 years) aging structural inspection to allow continuation of operation and eligibility of ARS.

- Limits of SLM applicability/methodology:
The approved applicability domain shall include any operational exclusions necessary to maintain validity of the fatigue accounting methodology.
The approved applicability domain shall be defined within the ICA.
SLM applicability shall be limited to CS-22 Utility Category.
Periods without valid recorded data shall conservatively revert to reference-spectrum fatigue accounting (e.g. missing flight data, invalid recordings, recorder malfunction, calibration non-conformity, interrupted traceability).
Periods during which the damage accumulation calculation method cannot provide sufficiently accurate fatigue accounting shall also revert to reference-spectrum fatigue accounting.