|  | AFMS/AMMS | SLM-AFMS-AMMS |
| --- | --- | --- |
|  |  | Edition :1 Rev: 0 |
|  |  | Date:06/06/2026 |
|  | CS-STAN 104b | Page: 1/21 |

# SLM for Glider Life Extension
Demonstration Project

A/C : GENERIC

Version : -

S/N : -

Registration : -

This document includes the Aircraft Flight Manual Supplement (AFMS) and the Aircraft Maintenance Manual Supplement for SLM operations.
It must be attached to the approved AIRCRAFT FLIGHT MANUAL (AFM) kept on board the aircraft.
The maintenance-related sections shall also be included in, or referenced by, the aircraft maintenance documentation.

## LIST OF EFFECTIVE PAGES

| Page | Edition — Revision — Date | Page | Edition — Revision — Date |
| --- | --- | --- | --- |
| 1 | Ed 1 Rev 0 06/06/2026 | 2 | Ed 1 Rev 0 06/06/2026 |
| 3 | Ed 1 Rev 0 06/06/2026 | 4 | Ed 1 Rev 0 06/06/2026 |
| 5 | Ed 1 Rev 0 06/06/2026 | 6 | Ed 1 Rev 0 06/06/2026 |
| 7 | Ed 1 Rev 0 06/06/2026 | 8 | Ed 1 Rev 0 06/06/2026 |
| 9 | Ed 1 Rev 0 06/06/2026 | 10 | Ed 1 Rev 0 06/06/2026 |
| 11 | Ed 1 Rev 0 06/06/2026 | 12 | Ed 1 Rev 0 06/06/2026 |
| 13 | Ed 1 Rev 0 06/06/2026 | 14 | Ed 1 Rev 0 06/06/2026 |
| 15 | Ed 1 Rev 0 06/06/2026 | 16 | Ed 1 Rev 0 06/06/2026 |
| 17 | Ed 1 Rev 0 06/06/2026 | 18 | Ed 1 Rev 0 06/06/2026 |
| 19 | Ed 1 Rev 0 06/06/2026 | 20 | Ed 1 Rev 0 06/06/2026 |
| 21 | Ed 1 Rev 0 06/06/2026 |  |  |

LIST OF EDITION/REVISION INCORPORATED

| Edition | Revision | Date | Description | Page |
| --- | --- | --- | --- | --- |
| 1 | 0 | 06/06/2026 | Original SLM AFMS AMMS |  |

### Scope

The recorder installed in the glider records in-flight acceleration data used to derive load-factor variations. These data are processed as part of the Structural Life Monitoring process to establish the measured usage spectrum of the individual glider and calculate its Equivalent Fatigue Index (EFI). The EFI is compared with the approved reference EFI basis. When the applicable EFI, inspection, and continued-airworthiness criteria are satisfied, the process may support operation beyond the originally certified safe-life limit.

### Limitations

This supplement does not introduce new aircraft flight-envelope limitations. However, SLM credit is only applicable within the approved SLM applicability domain defined in the approved continued-airworthiness and operating instructions.
Operation beyond the certified safe-life limit is permitted only when the approved SLM process, recorder status, calibration requirements, EFI status report, inspections, and Go/No-Go criteria are satisfied.

Before the certified safe-life limit, flights without valid SLM data receive no measured-usage credit and are accounted for in accordance with the approved data-processing procedure. Beyond the certified safe-life limit, the SLM recorder, required calibration status, validated EFI status report, and required inspections are conditions for continued airworthiness.

### Emergency procedures

There is no emergency procedure associated with this supplement.

### Normal procedures

### Recorder Description

The recorder is installed in the glider using a specific support (see installation instructions).
The USB-C connector should always be connected to the recorder.
All recorder controls are performed using the touch screen, the POWER/CLR and the RECORD buttons.

### Recorder Power Up

The recorder starts when the POWER/CLR button is pressed momentarily or when USB-C power is available.
There should be a green READY message at the bottom of the display.

If Amber or Red error messages are displayed at the bottom of the display, they should be corrected before flying (see message section).

If the recorder does not automatically start when glider battery is connected, check USB-C and power connection.

When there is no interaction with the recorder for more than 10 seconds, the display goes to stand-by mode to save battery power. Display can be activated by touching the screen.

### Recorder Power Down

The recorder stops when the POWER/CLR button is pressed for ~3 seconds or when USB-C power is removed.
To protect the battery, the recorder also stops when the battery level is below 5%.
When the recorder is in recording mode:
- if USB-C power is lost, the recorder continues to record.
- it will stop recording and power down only if the POWER/CLR button is pressed or the battery level is below 5%.

### Start Recording Mode

To start the recording mode, the user needs to press ~1 second on the RECORD button.

The recorder should display STARTING and immediately after, RECORDING.

A new file is created each time the recorder is activated, regardless of whether the glider has flown or not.

### Stop Recording Mode

The recording mode is stopped by:
- pressing on the RECORD button for ~3 seconds
- pressing the POWER/CLR button for ~3 seconds *
- if the battery level goes below 5% *
* : in this case the recorder will also power down.

### Pre-Flight Check

### When the glider's battery is installed and connected, the recorder should automatically start and the display should be active.

### Before Flight

Start recording mode.

/!\ If a flight is performed without valid SLM recording, or if the recorded data are invalid, the flight shall be treated in accordance with the approved missing-data procedure.
No measured-usage credit is granted for that flight.

### After flight

### Stop the recording mode.

There should be a new file logged in the recorder file system.
The file name is the registration of the glider followed by the date and time when the recording started.

### At the end of flying day

The last pilot-in-command to fly the glider must update the glider's usage information on the Structural Life Monitoring (SLM) website.

Using a device capable of WIFI connection (a smart phone or a PC), the files should be first downloaded from the recorder to the device and then transferred to the SLM website.

See section 6.3 for access to the recorder Web pages.

The files are listed on the main page of the recorder web server.

Selecting Download will transfer the selected file from the recorder to the device.

When a file is transferred to the device, it is necessary to upload it to the main SLM server using:
https://planeur-la-llagonne.com/gestion-vols/index.php

You need an account and a password to access the SLM server (contact support to get an account)

When the account and password are entered, you have access to the main SLM server page



To upload a file to the SLM server, you need to enter the date, the pilot name and select a file previously downloaded from the recorder to  the device.
When the file is correctly uploaded to the SLM webserver, it can be deleted from the recorder by selecting Delete on the SLM Recorder web page.

/!\ The recorder only accepts 50 files.

When all operations associated with the recorder are finished, the recorder should be stopped.

- Messages

| Message | Type |  | Operator action |  |
| --- | --- | --- | --- | --- |
| BOOT | Startup transient |  |  |  |
| READY | normal operation |  |  |  |
| RECORDING | normal operation |  |  |  |
| STARTING | recording start transient |  |  |  |
| STOPPING | recording stop transient |  |  |  |
| SHUTDOWN | Shutdown transient |  |  |  |
| TOUCH TO ACTIVATE | Recorder on stand-by |  | touch screen to activate recorder display |  |
| SD OK/CLR | SD recovered / clear prompt |  | press clear to confirm SD error correction |  |
| NEED SETTINGS | setup required |  | perform date, time, registration and password SETTINGS |  |
| ACC CAL REQ | accelerometer calibration required |  | perform Web acceleration calibration menu (START WIFI) |  |
| INST CAL REQ | installation calibration required |  | Perform Web installation calibration menu (START WIFI) |  |
| NO SD | SD not detected warning |  | insert a valid SD card |  |
| SD LOW | SD memory low |  | insert an SD card with enough free space (500 MB min.) |  |
| SD FULL (FILES) | SD has too many files (50 max.) |  | download/delete files from recorder Web interface (START WIFI) |  |
| LOW BATT | Low power warning |  | connect USB |  |
| BATTERY / LOW RECHARGE WITH USB | power too low |  | connect USB, shown for 10 seconds before shutdown |  |
| CAL FAULT | calibration fault |  | perform factory reset and perform new calibrations |  |
| FATAL WDG/CLR | watchdog fault acknowledgement |  | press the power/clear button to acknowledge/clear error * |  |
| ACCEL ERR | hardware/runtime error |  | press the power/clear button to stop recorder, then restart * |  |
| RTC ERROR | hardware/runtime error |  | press the power/clear button to stop recorder, then restart * |  |
| PMU ERROR | hardware/runtime error |  | press the power/clear button to stop recorder, then restart * |  |
| REC FAIL | recording failure |  | press the power/clear button to stop recorder, then restart * |  |
| TOUCH ERROR | hardware/runtime error |  | press the power/clear button to stop recorder, then restart * |  |
| ERROR | generic fallback error |  | press the power/clear button to stop recorder, then restart * |  |
| SD ERROR | SD/storage fault |  | press the power/clear button to stop recorder, then restart * |  |
| GENERIC ERROR | fatal error |  | press the power/clear button to stop recorder, then restart * |  |
| * It might require pressing the power/clear button up to 8 seconds to stop device. * If the error persists after restart, report problem to support |  |  |  |  |

### Recorder Special Functions

### Reset settings

/!\ when resetting the device, all parameters are lost.

To reset all device settings, press the RECORD button and immediately press the POWER/CLR button until the device stops.

When the device is restarted, it should ask the user to configure device settings and perform recorder and installation calibrations.
To help the user navigate through the SETTINGS pages, the required button are in Amber color.

### Recorder settings

/!\ the recorder cannot be operated until all settings and calibrations have been performed.

From the main page with the device ready, select MENU and SETTINGS.
The SETTINGS page allows user to set DATE, TIME, REGISTRATION and WIFI PASSWORD.

The values are adjusted using the rollers.
When values are as required, select SAVE.
If you are on a setting page by mistake, select BACK

### Access to the recorder Web pages

Using a device capable of WIFI connection (a smart phone or a PC), the calibration pages are accessed from the SLM website. 
To connect to the recorder WIFI and Website, select MENU and START WIFI.

Then open a browser on the device and type http://192.168.4.1/

When the WIFI activities are finished, you need to stop the WIFI by selecting the STOP WIFI or BACK buttons.

  - Initial calibrations

/!\ Access to the calibration pages requires a password.

Access the recorder Web page.

Select the Calibration menu.

Enter the password and select Unlock Calibration

### Recorder calibration

Select the calibration menu then the Accelerometer Calibration.

Install the recorder in its calibration support.

Start the calibration process and follow the workflow.
When all six faces have been calibrated, select SAVE.
If there is a problem, select CANCEL and start the process again.

The date of the recorder calibration should be entered in the aircraft logbook

### Installation calibration

Select the calibration menu then the Installation Calibration.

The glider should be positioned in the flight level attitude, as described in the Aircraft Maintenance Manual.
Start the calibration process and follow the workflow.
When calibration is correct, select SAVE.
If there is a problem, select CANCEL and start the process again.
The date of the installation calibration should be entered in the aircraft logbook.

### Recorder Installation

When all the installation steps have been performed, the installation details should be entered in the aircraft logbook. Date of installation.
Hardware and software versions.
Aircraft time at the installation date.

Before starting the installation, it is recommended to reset the settings and configure the recorder with the appropriate date, time, aircraft registration and WIFI password in the SETTINGS menu.

### Installation of recorder in glider

The recorder is provided with an installation support.
The support should be installed in the fuselage, in a rigid area close to the C.G. and wing spar, with suitable access for operation and routing of the USB-C cable (both the buttons and the USB-C socket are located on the right side of the recorder).

/!\ the recorder should not be in an area where there is a risk of object touching it.

  - Electrical wiring and power supply installation

The recorder is provided with a small power supply.
The power supply electrical wires should be connected to the glider’s battery connector and protected with a 1A fuse.
The power supply USB-C line should be connected to the recorder USB-C socket.
Everything should be secured to ensure there is no electrical risk. The USB-C line should also be secured close to the recorder to make sure it cannot introduce vibration on the recorder.

  - Initial recorder calibration

Before installing the recorder in the glider on its support, it should be calibrated. See 6.4.1.

### Installation calibration

When the recorder is installed in the glider, it should be calibrated. See 6.4.2.

### Maintenance
During the annual visit and before ARC renewal, a recorder calibration must be carried out and the status report of the Equivalent Fatigue Index (EFI) usage statistics downloaded from the SLM Web site.
The calibration results are a condition for validating the recordings uploaded since the previous calibration.

### The EFI status report provides the information required to allow airworthiness renewal and aircraft return to service.
The date and reference of the EFI status report shall be logged in the aircraft logbook and the status included in the aircraft maintenance documentation.
The glider Aircraft Maintenance Program should be updated to reflect these requirements.

### Calibration

### The calibration shall be performed at least once a year but can be repeated as many times as desired.
The process to calibrate is described in paragraph “6.4.1”

### Equivalent Fatigue Index documents

/!\ The recorder calibration should be performed before generating the EFI status report. Flights performed after the last valid calibration and before the EFI report may be treated as non-validated or handled in accordance with the approved data-processing procedure.

The EFI report provides the information required for the Go/No-Go decision supporting Airworthiness Review Certificate renewal and release to service.

### Firmware Update

/!\ Firmware update should only be attempted if instructed by SLM support

Access the recorder Web page.

Select Firmware Update

The new firmware should be provided by SLM support and uploaded to the device which is used to connect to the recorder.
Select the Browse button and select the firmware file which is on the device.
Select Upload Firmware and wait for device to be updated.
The device shall restart after firmware update.
It is possible that settings and calibrations have to be performed.
The date and reference of the firmware should be entered in the aircraft logbook.

### Performance

There are no performance changes associated with this supplement

- Weight and balance

Due to very light weight and location, there are no changes associated with this supplement

- In case of problem

Contact support :
+33 783232334
jeanluc@derouineau.com