|  | METHODOLOGY REPORT | SLM-MR |
| --- | --- | --- |
|  |  | Edition: 1 Rev: 1 |
|  |  | Date: 07/08/2026 |

Structural Life Monitoring
for Composite Gliders

Methodology Report

This document describes the methodology used to assess and justify extended operational life for composite gliders using Structural Life Monitoring (SLM). The method calculates an Equivalent Fatigue Index (EFI) from onboard acceleration measurements and compares the cumulative operational EFI with the reference EFI calculated from the Kossira & Reinke Utility spectrum.

Based on: "Extension of Glider Life using Structural Life Monitoring" - J.L. Derouineau, FFVP / EGU [7].

LIST OF INCORPORATED EDITIONS/REVISIONS

| Edition | Revision | Date | Description | Page |
| --- | --- | --- | --- | --- |
| 1 | 0 | 05/07/2026 | SLM Methodology Report original | 1-28 |
| 1 | 1 | 07/08/2026 | Update following experts review | 1-26 |

TABLE OF CONTENTS

1. Purpose and Scope	4

2. Regulatory Basis	5

3. Key Principles	6

4. Methodology	8

4.1 Reference Load Spectrum	8

4.2 Real Load Spectrum Acquisition	9

4.3 Load Spectrum Processing — Kossira & Reinke Counting Method	9

4.4 Equivalent Fatigue Index — Palmgren-Miner Rule	10

4.5 Life Extension Assessment	11

5. Applicability and Limitations	13

6. Concept of Operation associated with methodology	14

7. Appendix A — Kossira & Reinke Processing Rules for Software Implementation	15

A.1 Constants, class convention, and notation	15

A.2 Load factor to 1024-class conversion	16

A.3 Hysteresis filtering in 1024-class space	16

A.4 Reduction from 1024 classes to 32 classes	16

A.5 Peak and valley extraction	16

A.6 Raw Markov transition matrix	17

A.7 Fixed Kossira & Reinke reference-load convention	17

A.8 Markov matrix transformation into exceedance occurrences	18

A.9 Required Kossira & Reinke processing outputs	18

A.10 Kossira & Reinke processing verification checks	19

8. Appendix B — EFI Calculation Rules for Software Implementation	20

B.1 Required inputs	20

B.2 Fixed reference level used by counting and EFI	20

B.3 Reference spectrum scaling	20

B.4 Reference normalization constant	21

B.5 Number of cycles to failure	21

B.6 Monitored and total EFI	21

B.7 Life Ratio	21

B.8 Life-extension estimate	21

B.9 Valid duration reporting	22

B.10 Required EFI outputs	22

9. Appendix C — ABB.163-Derived Utility Reference Spectrum	23

C.1 Source material and import convention	23

C.2 Load-factor class scale	23

C.3 Generated ABB.163 Utility reference spectrum	24

C.4 Validation against ABB.164 by Kossira & Reinke (K&R) class	24

C.5 Conclusion	25

10. References	26

# 1. Purpose and Scope

This document describes the methodology used to assess and justify extended operational life for composite gliders using Structural Life Monitoring (SLM). The method calculates an Equivalent Fatigue Index (EFI) from onboard acceleration measurements and compares the cumulative operational EFI with the reference EFI calculated from the Kossira & Reinke Utility spectrum.

The methodology applies to composite gliders operated within the EASA CS-22 Utility-category flight envelope and for which an approved Service Life Limit, denoted L, has been established. Aerobatic-category operation and operation outside Utility-category limitations are outside the initial SLM scope.

The numerical Utility reference spectrum used by the EFI calculation is generated from the published ABB.163 Utility Markov matrix using the Kossira & Reinke Markov-matrix-to-occurrence-spectrum transformation. ABB.164 is used as the published graphical check showing that the ABB.163-derived spectrum reproduces the original Kossira & Reinke Utility curve [2] [8].

SLM does not seek to demonstrate additional intrinsic fatigue capability beyond that accepted during Type Certification. The original structural substantiation, including its assumptions, margins, inspections and conservatism, is not reopened or reinterpreted.

SLM compares the cumulative equivalent fatigue exposure of the aircraft, expressed as EFI, with the reference EFI associated with the approved Service Life Limit L. This reference EFI is normalized to:

EFIref,L=1

Continued operation beyond L may be justified while the cumulative aircraft EFI remains below this reference value and the applicable continued-airworthiness provisions are satisfied.

# 2. Regulatory Basis

The method is consistent with the following regulatory framework:

CS 22.627 (Fatigue Strength): requires the structure, as far as practicable, to avoid stress concentrations where variable stresses above the fatigue limit are likely to occur in normal service. It does not establish a service-life substantiation method or a universal Service Life Limit.

EASA CM-S-006: recognizes the use of Kossira & Reinke load spectra as a reference basis for fatigue analysis and testing of sailplanes.

CS-STAN / Part ML: governs installation of onboard recorders and maintenance requirements.

CS 22.1529 / CS 22.1585: Instructions for Continued Airworthiness, flight and maintenance.

For the SLM method, L is the Service Life Limit applicable to the aircraft under the approved type-design and continued-airworthiness data. SLM does not assume that one universal numerical life limit applies to all composite gliders.

Different approaches have been used for the service-life justification of composite gliders, including full-scale fatigue testing and static overload testing. SLM proposes an additional measured-usage-based approach for ageing-glider life management. Its acceptability and associated substantiation requirements remain to be agreed with the competent authority through the applicable certification process.

Similar measured usage principles have been used in aviation within structural health and usage monitoring programmes, including Individual Aircraft Tracking, particularly in military aviation. SLM does not attempt to reproduce those approaches directly or claim that they are directly transferable to gliders. The relevant precedent is the general principle that measured usage can be compared with a reference-usage assumption, provided the method is controlled, reproducible and supported by appropriate inspections.

# 3. Key Principles

Service Life Limit and SLM reference-exposure limit

For the aircraft under consideration, let L be the approved Service Life Limit. This value has already been accepted through the applicable Type Certification substantiation, using whatever combination of test, analysis, assumptions, margins, conservatism and inspections was applicable.

SLM does not attempt to reopen or reinterpret that substantiation. The only reference level required by SLM is the fatigue exposure associated with the approved reference spectrum over L flight hours.

EFI is used as a relative exposure index, not as a direct estimate of physical damage. Its absolute numerical normalization is therefore arbitrary, provided the same normalization is applied consistently to the reference and measured spectra. The approved reference exposure over L is assigned an EFI value of unity:

EFIref,L=1

Operational exposure is evaluated against that same reference level. It comprises the monitored EFI calculated from valid aerodynamic data and the unmonitored EFI assigned at the reference rate for periods not supported by valid aerodynamic measurements:

EFItotal=EFImonitored+EFIunmonitored

The allowable reference-exposure condition is:

EFItotal<EFIref,L=1

This does not imply that the physical structure is expected to fail when EFI=1, nor that the true accumulated material damage is known. It means only that the operational exposure calculated using the same counting method and fatigue-severity assumptions has not exceeded the approved reference exposure associated with L.

Any safety margin, test factor or other conservatism used in the original certification substantiation remains embedded in the approved value of L. SLM neither removes nor attempts to quantify that margin.

SLM as a reference-spectrum comparison method

SLM is not certification by analysis and does not replace the structural substantiation performed during Type Certification. It does not attempt to determine the absolute fatigue life of a composite structure, identify the critical structural location, or predict the critical composite failure mode.

The method has a more limited objective: it uses the already accepted reference exposure over L and compares it with the operational exposure of an individual glider. The Palmgren-Miner calculation is therefore used as an Equivalent Fatigue Index, calculated consistently for the reference spectrum and for the measured spectrum.

Relationship between measured load factor and wing fatigue loading

The recorder-measured vertical load factor Nz is used as the practical first-order input for reconstructing the operational load spectrum. It is a proxy for wing fatigue loading rather than a complete measurement of structural stress or wing-root bending moment.

During aerodynamic flight, normalized wing-root bending moment and aircraft vertical load factor are closely related. However, the wing lift required to produce a given aircraft load factor can also be influenced by the tail force required to balance pitching moment. Because the tail aerodynamic force is normally small relative to the main-wing lift, its effect on wing loading is expected to be of secondary order. The effect is also not unidirectional: depending on the manoeuvre and pitching-moment balance, the tail force may require the wing to produce either more or less lift than would be inferred from Nz alone.

The present SLM methodology does not correct measured Nz for this effect. Under the adopted first-order model, a constant proportionality between load-factor amplitude and stress amplitude is absorbed in the common reference-versus-measured calculation and cancels in the life ratio. This cancellation does not remove uncertainty caused by condition-dependent effects such as tail balancing load.

The first-order relationship is applicable only during aerodynamic flight. It does not hold during non-aerodynamic phases such as takeoff roll, landing roll, or winch launch. These phases shall therefore be excluded from the measured aerodynamic spectrum. The measured Kossira & Reinke occurrence spectrum is generated only from valid aerodynamic recording time. Periods excluded from the measured spectrum receive no measured-usage credit and are assigned EFI at the reference rate. The same treatment applies to time accumulated before valid SLM measurements begin and to missing or invalid data sequences.

High Cycle Fatigue regime

Glider structures are designed so that operational stress levels remain well within the elastic domain of materials. The structure is therefore considered to operate in the High Cycle Fatigue (HCF) regime, which justifies the use of a Basquin-type S/N curve representation for the conservative EFI calculation used in this methodology.

# 4. Methodology

## 4.1 Reference Load Spectrum

The reference load spectrum used by the generic SLM methodology is the Kossira & Reinke Utility spectrum [2][8]. EASA CM-S-006 recognizes the Kossira & Reinke spectra as a reference basis for the fatigue analysis and testing of sailplanes. Their use therefore provides a legitimate common representation of glider operational usage for the reference-versus-measured comparison performed by SLM.

In the original Kossira work, the spectrum parameter is the normalized wing-root bending moment:

n=MbrMbr, 1g-flight

where Mbr, 1g-flight is the wing-root bending moment during steady 1g flight. During normal aerodynamic flight, this normalized bending-moment parameter is approximately represented by the aircraft vertical load factor Nz. The SLM methodology therefore uses measured Nz as the practical equivalent parameter for reconstructing the operational spectrum.

The Kossira & Reinke spectrum was derived from measured glider operations and subsequently extrapolated to establish an envelope-covering reference spectrum. Figure 12 of Kossira [2] shows that the resulting cumulative-frequency distribution is broadly comparable with the Thielemann/Franzmeyer load spectrum over most of the normalized-load range.

The extrapolation method nevertheless tends to slightly increase the cumulative number of occurrences in the extreme positive-load region, particularly above approximately n=5, and to extend the upper end of the spectrum. The absolute occurrence counts in this region are very low. This characteristic is acknowledged as part of the construction of the Kossira & Reinke reference spectrum and does not alter the rationale for its use by SLM.

The spectrum remains an EASA-recognized, documented and reproducible reference representation of glider usage. SLM therefore retains the published Utility spectrum without adjustment and applies equivalent Kossira & Reinke counting rules, class definitions and Markov-matrix processing to the reference and measured operational data.

For the approved SLM calculation package, the Utility reference occurrence table shall be generated from the published ABB.163 Utility Markov matrix using the controlled transformation defined in Appendix A.8. This ABB.163-derived occurrence table is the operational reference input for EFI.

The ABB.163-derived table is verified against ABB.164, which provides the original published graphical occurrence curve. ABB.164 is not used as a digitized numerical reference table.

The reference occurrence table is normalized to 6,000 flight hours. For calibration of the SLM reference exposure, the reference occurrences are scaled continuously to the applicable Service Life Limit L.

The ABB.163-derived Utility spectrum is used to normalize the reference exposure over L, to calculate the reference EFI over durations matched to the measured data, and to define the reference EFI rate applied to periods not supported by valid aerodynamic measurements.

## 4.2 Real Load Spectrum Acquisition

The vertical load factor Nz is recorded continuously during flight using a tri-axial accelerometer located as close as possible to the Center of Gravity, in a rigid area near the wing spar. Proximity to the CG minimizes contamination from centripetal accelerations during attitude changes, while a rigid mounting area avoids local structural flexibility corrupting the measurement. Atmospheric turbulence and trajectory induce load variations at relatively low frequency, typically below 10 Hz (CS 22.341).

Accelerometer requirements:

- Minimum sampling rate: 20 Hz

- 6-face calibration (using gravity as a reference) at each annual inspection to ensure recorder performance

- Installation calibration with the glider in level-flight attitude and wings level, so that the recorder-corrected vertical axis is aligned with the gravity vector in the reference flight attitude

- Data is stored on onboard electronic memory and uploaded to a central repository.

Non-flight phases. During takeoff roll, landing roll, winch launch, and any other phase dominated by non-aerodynamic forces, the proportionality between vertical load factor and structural stress no longer holds. The accelerometer signal from these sequences shall therefore not be used to build the real aerodynamic load spectrum. These phases receive no measured-usage credit and their duration is assigned EFI at the reference rate.

## 4.3 Load Spectrum Processing — Kossira & Reinke Counting Method

Real measured spectra are generated using the Kossira & Reinke counting method [2][8]. The same class table, filtering rule, Markov-matrix convention, fixed reference-load convention and exceedance transformation are used for measured data and for the ABB.163-derived reference spectrum.

The processing chain for real measured data shall be:

- validate and select the aerodynamic flight samples to be credited by SLM;

- convert each valid load-factor sample into a 1024-class representation;

- apply the Kossira & Reinke hysteresis filter in 1024-class space (a standard filter that discards small load-factor wiggles too small to count as real fatigue cycles);

- convert the retained filtered values into 32-class load-factor values;

- remove repeated consecutive 32-class values;

- extract the alternating peak/valley sequence;

- build the 32×32 raw Markov transition matrix (a table counting how often the load factor moves from each class to each other class, the standard way fatigue engineers count load-reversal cycles);

- use the fixed Kossira & Reinke reference level LF0=1.0 and its corresponding class to split the Markov matrix into upper and lower exceedance branches;

- transform the raw Markov matrix into upper and lower exceedance occurrences;

- produce the 32-class occurrence spectrum;

- compute EFI using §4.4 and Appendix B.

Appendix A defines the Kossira & Reinke software rules, Appendix B defines the EFI calculation rules, and Appendix C documents the ABB.163-derived reference spectrum and its validation against ABB.164.

## 4.4 Equivalent Fatigue Index — Palmgren-Miner Rule

The Palmgren-Miner rule provides a linear cumulative index for fatigue. In classical notation this value is often denoted D, for damage. In the present SLM method, the same mathematical quantity is denoted Equivalent Fatigue Index, EFI, to avoid implying that the calculation represents measured physical damage in the composite structure:

EFI=i​niNi

where ni is the number of occurrences at load-factor level i, and Ni is the equivalent number of occurrences to failure associated with that level under the selected fatigue model.

For SLM, the Palmgren-Miner rule is used as a consistent comparison framework between the reference and measured spectra. The absolute value of EFI shall not be interpreted as a direct measurement of physical composite damage at a specific structural location.

Application of the Palmgren-Miner rule requires an equivalent number of occurrences to failure, Ni, for each load-amplitude class. In the High Cycle Fatigue regime, the Basquin model represents the relationship between stress amplitude and fatigue life using a power law:

σa=ANb

where σa is the stress amplitude, N is the number of occurrences to failure, A is a material-dependent constant, and b is the Basquin exponent. Solving for N gives:

N=σaA1/b

Under the first-order SLM assumption that stress amplitude is proportional to load-factor amplitude, the equivalent number of occurrences to failure for class i may be written:

Ni=C LFi-LF01/b

where C combines the Basquin constant and the proportionality between stress amplitude and load-factor amplitude.

To simplify the notation, the positive EFI exponent is defined as:

k=-1b

With this notation:

Ni=C LFi-LF0-k

Let nj,ref,L be the number of occurrences at class j in the reference spectrum scaled to L flight hours, and let LF0=1.0 be the fixed Kossira & Reinke reference level. The SLM calculation is normalized by assigning an EFI value of unity to the reference exposure over L:

1=EFIref,L=j​nj,ref,LNj=1Cj​nj,ref,LLFj-LF0k

The normalization therefore determines C directly:

C=j​nj,ref,LLFj-LF0k

The constant C is not an independently selected material parameter. It is fixed by the reference spectrum, the selected exponent and the normalization EFIref,L=1.

For any spectrum x, reference or measured, the corresponding EFI is:

EFIx=1Ci​ni,xLFi-LF0k

This formulation gives EFIref,L=1 by construction, uses the same fixed LF0 for both the reference and measured spectra.

For the generic SLM methodology, the proposed Basquin parameter is:

b=-0.16

and therefore:

k=6.25

The selected value is derived from published fatigue data for composite material systems [9] [10] [11] [12] [13]. Although the supporting data are not specific to glider wing, they cover GFRP/CFRP epoxy systems representative of materials commonly used in glider primary structures. The resulting load-amplitude weighting is therefore considered a reasonable conservative lower-bound representation for typical composite glider structures.

The selected exponent is not intended to reproduce the precise S/N curve, stress ratio, material system or failure mode of a particular wing spar. It is applied consistently to the reference and measured spectra as part of a relative exposure comparison, rather than an absolute fatigue-life prediction. Additional representative glider-structure data may be used to confirm or refine the generic value if such data become available.

## 4.5 Life Extension Assessment

The Life Ratio LR quantifies how much faster or slower the measured operation consumes the approved reference exposure:

LR=EFIref,tEFIreal,t

where both values are computed over the same observed valid aerodynamic duration treal. Here EFIref,t is the reference-spectrum EFI normalized to treal and EFIreal,t is the measured EFI over the same duration.

To evaluate LR in practice, the reference and measured spectra must cover the same exposure time. The published reference spectrum, normalized to 6,000 flight hours, is scaled down to treal. The conservative integer-floor convention may be applied to the scaled reference occurrences because the real observation period is much shorter than 6,000 hours. Using the common fixed reference level LF0 and exponent k:

LR=i​⌊ni,ref,6000 treal6000⌋LFi-LF0ki​ni,realLFi-LF0k

The same normalization constant C applies to both spectra and cancels from the ratio. The ratio is therefore independent of the arbitrary EFI normalization.

The estimated additional life beyond the approved Service Life Limit is:

textension=LR-1 tremaining,ref

where tremaining,ref is the flight time remaining until L under reference-spectrum assumptions.

The projected life extension assumes that future operation remains reasonably consistent with the accumulated measured usage. LR, cumulative EFI and projected EFI are therefore recalculated at each approved review point using the accumulated valid data. The Life Ratio is highly sensitive to occasional high-load manoeuvres; consistently severe operation may yield little or no life extension. Favourable results observed during the experimental programme do not constitute a guarantee for all aircraft or usage profiles.

# 5. Applicability and Limitations

- The method applies to composite gliders operated within their certified Utility-category flight envelope. Aerobatic-category operation and operation outside Utility-category limitations are not credited under the initial SLM scope.

- The Kossira & Reinke Utility spectrum is used as the common EASA-recognized reference representation of glider usage. It is an extrapolated, envelope-covering spectrum. Its extrapolation slightly increases the low-frequency extreme positive-load tail, particularly above approximately n=5, but the resulting distribution remains broadly comparable with other published glider spectra and is retained without adjustment.

- Measured Nz is a first-order proxy for normalized wing-root bending moment and wing fatigue loading, not a direct structural-stress measurement. The effect of tail balancing load is not explicitly corrected. It is expected to be secondary because the tail force is small relative to main-wing lift and may either increase or reduce wing loading, depending on the manoeuvre and pitching-moment balance.

- The Basquin parameter b=-0.16 is a generic conservative lower-bound severity-weighting parameter for typical GFRP/CFRP epoxy glider structures. It is applied consistently to the reference and measured spectra and is not presented as the exact fatigue property of a particular wing spar.

- All operational time not supported by valid aerodynamic SLM measurements is assigned EFI at the reference rate. This includes time accumulated before valid SLM monitoring begins, excluded non-aerodynamic phases, and missing or invalid data sequences.

- A favourable Life Ratio is not guaranteed. Severe operation or repeated high-load manoeuvres may result in little or no life extension.

- SLM does not assess or claim additional intrinsic material capability beyond the approved Service Life Limit.

- Continued operation remains subject to the Instructions for Continued Airworthiness defined in the approved AFMS-AMMS (Aircraft Flight Manual Supplement / Aircraft Maintenance Manual Supplement), including recorder calibration, data validation, inspections and all other continued-airworthiness requirements.

# 6. Concept of Operation associated with methodology

This section provides the methodology-level Concept of Operation (CONOPS). Practical installation, calibration, data acquisition, data validation, processing, maintenance, inspection, reporting, and airworthiness decision rules are controlled by the certification package, including the Certification Programme and the AFMS-AMMS.

Phase 1 — Instrumentation and Data Collection (below the approved Service Life Limit) The SLM recorder is installed in accordance with the approved installation basis. Installation calibration and annual 6-face calibration are recorded in the aircraft logbook. Flight data are uploaded to the controlled SLM repository and processed using the approved toolset. Reference-rate substitution is applied to all operational time not supported by valid aerodynamic SLM measurements, including time accumulated before valid SLM monitoring begins, excluded non-aerodynamic phases, and missing or invalid data sequences. No measured-usage credit is granted for these periods.

Phase 2 — EFI Evaluation at the Approved Service Life Limit When the aircraft reaches L, a structural inspection is performed in accordance with the approved AFMS-AMMS. Continued operation may be authorized when the inspection result is acceptable and the approved EFI assessment shows:

EFItotal<1

Phase 3 — Operation Beyond the Approved Service Life Limit Operation beyond L continues with SLM monitoring and the approved continued-airworthiness controls. A Go/No-Go decision is made at each annual visit, ARC (Airworthiness Review Certificate) renewal, or other approved review point using the validated SLM EFI status report. The Go criterion includes both the current cumulative EFI and the projected EFI consumption before the next approved review point:

EFItotal,projected<1

Recurring structural inspections continue at the interval defined in the approved AFMS-AMMS. Any existing approved type-specific inspection requirements remain applicable unless explicitly superseded by the approved SLM programme.

# 7. Appendix A — Kossira & Reinke Processing Rules for Software Implementation

This appendix defines the deterministic software rules used to generate a 32-class occurrence spectrum from valid aerodynamic load-factor data using the Kossira & Reinke method.

## A.1 Constants, class convention, and notation

The Kossira & Reinke processing chain uses 1024 fine classes and 32 coarse classes:

Nfine=1024

Ncoarse=32

For the Utility reference reconstruction, the load-factor range is taken from the ABB.164 axis:

LFmin=-4g

LFmax=+7g

The resulting range is:

LFrange=11g

This is consistent with a 10-bit acquisition or digitization hypothesis: the 11 g range is divided into 1024 fine classes. Therefore:

ΔLFfine=111024=0.0107421875g

Grouping 32 fine classes into one coarse Kossira & Reinke class gives:

ΔLFcoarse=32ΔLFfine=1132=0.34375g

The Kossira & Reinke hysteresis threshold is:

DX=10

therefore:

DXg=10ΔLFfine=0.107421875g

The approved Utility class table shall be included in the calculation package. In zero-based notation, coarse class c covers the interval:

LFmin+cΔLFcoarse≤LF<LFmin+c+1ΔLFcoarse

with:

c=0…31

For EFI calculation, the class representative value is the class center:

LFc=LFmin+c+12ΔLFcoarse

Equivalently, in one-based notation:

LFq=LFmin+q-12ΔLFcoarse

with:

q=1…32

where q=1 is the lowest load-factor class and q=32 is the highest load-factor class.

Software shall use zero-based class numbering internally unless otherwise specified:

fine classes: 0…1023;

coarse classes: 0…31.

## A.2 Load factor to 1024-class conversion

For each valid load-factor sample LF, the corresponding 1024-class value shall be:

q1024=clip⌊LF-LFminΔLFfine⌋,0,1023

where LFmin=-4g and clipx,0,1023 limits the value to the valid range.

The load-factor value associated with a fine class may be represented by the class center:

LFq1024=LFmin+q1024+12ΔLFfine

Samples clipped to the first or last fine class shall be retained according to the normal processing rules. Clipping events shall be counted and reported when detailed data-quality reporting is required.

## A.3 Hysteresis filtering in 1024-class space

The Kossira & Reinke filter shall be applied before reduction to 32 classes.

The first valid sample shall be retained as the first filtered value.

For each subsequent sample q1024n, the sample shall be retained only if:

q1024n-q1024,lastretained>DX

where q1024,lastretained is the last retained filtered value.

Samples for which:

q1024n-q1024,lastretained≤DX

shall be rejected as low-amplitude variation/noise for Kossira & Reinke counting.

The retained filtered sequence shall be stored as an auditable intermediate output when detailed processing evidence is required.

## A.4 Reduction from 1024 classes to 32 classes

Each retained 1024-class value shall be converted to a 32-class value using integer division:

q32=⌊q102432⌋

The load-factor value associated with a coarse class is the class center:

LFq32=LFmin+q32+12ΔLFcoarse

Repeated consecutive 32-class values shall be removed before peak/valley extraction, because they do not define a transition between different load-factor classes.

## A.5 Peak and valley extraction

The input to peak/valley extraction shall be the filtered 32-class sequence after removal of repeated consecutive values.

For three consecutive retained 32-class values a, b, and c:

b is a peak if b>a and b>c;

b is a valley if b<a and b<c.

The first and last values of the filtered 32-class sequence shall be retained when required to define the first or last transition of the processed segment.

The approved implementation shall define endpoint and plateau handling deterministically. The final peak/valley sequence shall not contain duplicate consecutive values. If the processed segment contains fewer than two peak/valley values, no Markov transition shall be generated.

The peak/valley sequence shall be stored as an auditable intermediate output when detailed processing evidence is required.

## A.6 Raw Markov transition matrix

The raw Markov transition matrix shall be a 32×32 integer matrix M.

The matrix convention shall be:

Mi,j=number of transitions from classjto classi

where:

column j is the “from” class;

row i is the “to” class.

For each consecutive pair in the peak/valley sequence:

pn→pn+1

the matrix cell shall be incremented as follows:

Mpn+1,pn=Mpn+1,pn+1

Diagonal transitions Mk,k shall be zero because repeated consecutive 32-class values are removed before peak/valley extraction and because a transition from a class to itself is not a peak/valley transition.

The total number of Markov transitions shall satisfy:

ijMi,j=NPV-1

where NPV is the number of values in the peak/valley sequence.

For a closed sequence, each class shall have equal arrivals and departures:

jMk,j=iMi,k

For an open sequence, the first class may have one additional departure and the last class may have one additional arrival. This imbalance shall be reported but shall not by itself invalidate the sequence.

## A.7 Fixed Kossira & Reinke reference-load convention

The Kossira & Reinke load parameter is normalized by the wing-root bending moment measured during steady wings-level 1g flight. The fixed reference level used for both the reference and measured spectra shall therefore be:

LF0=1.0

The same fixed value shall be used for:

splitting every raw Markov matrix into upper and lower exceedance branches; and

computing the load-factor amplitude term in the EFI calculation.

The corresponding zero-based 32-class index is:

k0=clip⌊LF0-LFminΔLFcoarse⌋,0,31

For the Utility class convention of Appendix A.1, LF0=1.0 lies in one-based class q=15 and zero-based class k0=14.

The arithmetic mean of the measured samples shall not be used for either the Markov-matrix split or the EFI amplitude calculation. It may be reported as an optional diagnostic but shall not affect the calculated occurrence spectrum or EFI.

## A.8 Markov matrix transformation into exceedance occurrences

A raw 32×32 Markov matrix shall be transformed into a 32-class occurrence spectrum by counting upward and downward exceedances relative to the fixed reference class k0 corresponding to LF0=1.0.

The matrix convention is:

Mi,j=number of transitions from class j to class i

For classes at or above the reference class, k≥k0:

Ok=i=k31j=0k-1Mi,j

For classes below the reference class, k<k0:

Ok=i=0kj=k+131Mi,j

With this convention, the reference class belongs to the upper branch. No smoothing, zeroing, or replacement of the reference-class occurrence shall be performed.

The occurrence spectrum Ok shall be stored as an auditable intermediate output when generated by the SLM software.

## A.9 Required Kossira & Reinke processing outputs

The software shall be capable of producing, for each real measured processed flight or data set:

valid sample count and processed duration;

1024-class filtered sequence;

32-class filtered sequence;

peak/valley sequence;

raw 32×32 Markov transition matrix;

fixed reference load factor LF0=1.0;

reference 32-class index k0;

upper and lower exceedance occurrence branches;

final 32-class occurrence table;

## A.10 Kossira & Reinke processing verification checks

The software shall verify and report the following processing-consistency checks for real measured data processed from raw samples:

- the sum of the raw Markov matrix equals the number of peak/valley transitions;

- diagonal Markov cells are zero;

- raw Markov row/column imbalance is consistent with the sequence start/end conditions.

# 8. Appendix B — EFI Calculation Rules for Software Implementation

This appendix defines the deterministic software rules used to compute EFI and associated life-extension quantities from the occurrence spectra generated by Appendix A.

## B.1 Required inputs

The EFI software shall use the following inputs:

- approved ABB.163-derived Utility reference occurrence table;

- applicable approved Service Life Limit L;

- real measured occurrence table generated by Appendix A;

- valid real aerodynamic recording duration associated with the measured occurrence table;

- unmonitored aircraft flight time used for the total-EFI assessment;

- Basquin exponent b and positive EFI exponent k=-1/b.

For the generic SLM methodology:

b=-0.16

k=6.25

The 6,000-hour Kossira & Reinke reference normalization is a fixed methodology input. The reference spectrum is scaled to the applicable value of L; no universal 12,000-hour life or universal fatigue safety factor is introduced into the EFI equations.

## B.2 Fixed reference level used by counting and EFI

The Kossira & Reinke processing and EFI calculation shall use the same fixed reference level for both the reference and measured spectra:

LF0=1.0

The arithmetic average of an individual measured sequence shall not be used for Markov-matrix splitting or EFI amplitude calculation.

## B.3 Reference spectrum scaling

The generic Utility reference occurrence table is normalized to 6,000 flight hours.

For establishment of the reference exposure over the approved Service Life Limit:

ni,ref,L=L6000ni,ref,6000

This scaling shall be continuous; integer flooring shall not be applied when establishing the reference normalization constant C over L.

For comparison with a valid measured observation duration treal, the reference occurrence table shall be scaled to the same duration. When the conservative integer-floor convention is used:

ni,ref,t=⌊ni,ref,6000treal6000⌋

## B.4 Reference normalization constant

The reference normalization constant shall be:

C=j​nj,ref,LLFj-LF0k

This normalization assigns:

EFIref,L=1

## B.5 Number of cycles to failure

For each load-factor class i, the equivalent number of occurrences to failure shall be:

Ni=CLFi-LF0-k

If LFi=LF0 exactly, the corresponding amplitude and EFI contribution are zero. Ni may be reported as infinite or not applicable to avoid division by zero in software output.

## B.6 Monitored and total EFI

The EFI accumulated from valid measured aerodynamic recordings shall be:

EFImonitored=1Ci​ni,realLFi-LF0k

The unmonitored aircraft flight time shall be conservatively accumulated at the reference rate:

EFIunmonitored=tunmonitoredL

The total aircraft EFI shall be:

EFItotal=EFIunmonitored+EFImonitored

The measured spectrum shall not be applied retroactively to unmonitored flight time.

## B.7 Life Ratio

For life-extension assessment over an observed valid aerodynamic recording duration treal, the Life Ratio shall compare the measured EFI with the reference EFI normalized to the same duration:

LR=EFIref,tEFIreal,t

Using the common normalization, this may be computed as:

LR=i​⌊ni,ref,6000treal6000⌋LFi-LF0ki​ni,realLFi-LF0k

when the conservative integer-floor convention is used.

## B.8 Life-extension estimate

The estimated additional life beyond the approved Service Life Limit shall be:

textension=LR-1tremaining,ref

where tremaining,ref is the time remaining until L under reference-spectrum assumptions.

## B.9 Valid duration reporting

The Kossira & Reinke and EFI software shall report the valid aerodynamic recording duration actually used in the calculation, together with the unmonitored duration used in the total-EFI assessment.

## B.10 Required EFI outputs

The software shall report:

- reference spectrum source;

- applicable Service Life Limit L;

- valid real aerodynamic recording duration;

- unmonitored duration used in the total-EFI calculation;

- Basquin parameter k;

- measured EFI;

- unmonitored EFI contribution;

- total EFI;

- Life Ratio LR;

- estimated life extension;

# 9. Appendix C — ABB.163-Derived Utility Reference Spectrum

This appendix documents the reference-data chain used by the SLM calculation:

ABB.163 Utility Markov matrix
→ Appendix A.8 Markov-matrix-to-spectrum transformation
→ ABB.163-derived Utility occurrence spectrum
→ EFI reference input

ABB.164 is used as the published graphical validation of the generated spectrum.

## C.1 Source material and import convention

The reference spectrum is generated from the published ABB.163 32×32 Utility Markov transition matrix [8].

The SLM class index increases with load factor: class 1 is the lowest (most negative) load-factor class and class 32 is the highest (most positive) load-factor class (Appendix A.1). For clarity in this import step, that ascending index is denoted qSLM below; it is the same index written q elsewhere in Appendices A and C.

ABB.163 prints its Markov matrix rows in the opposite order, from the highest to the lowest load-factor class. The printed row label, denoted qABB, is therefore mapped to the ascending SLM class index qSLM by:

qSLM=33-qABB

This reversal mapping is specific to the ABB.163 reference-data import: it only reorders the printed rows to match the SLM ascending convention and does not change the load-factor value or width associated with any class. Real measured data are generated directly in the ascending qSLM convention of Appendix A and never require this mapping.

## C.2 Load-factor class scale

ABB.164 provides the load-factor scale associated with the published occurrence spectrum. The graph axis spans approximately:

-4g to+7g

therefore:

LFrange=11g

Kossira & Reinke use 1024 fine classes, which is consistent with a 10-bit acquisition or digitization scale. Dividing the 11 g range into 1024 fine classes gives:

ΔLFfine=111024=0.0107421875g

Each 32-class spectrum bin contains 32 fine classes, therefore:

ΔLFcoarse=32ΔLFfine=1132=0.34375g

The Kossira & Reinke hysteresis threshold is therefore:

DXg=10ΔLFfine=0.107421875g

The class representative value used for EFI is the class center:

LFq=-4.0+q-120.34375

with:

q=1…32

The fixed reference load factor used for the ABB.163 transformation and EFI is:

LF0=1.0

This lies in one-based class q=15 and zero-based class k0=14; therefore classes q=1…14 form the lower branch and classes q=15…32 form the upper branch for the Utility reference transformation.

## C.3 Generated ABB.163 Utility reference spectrum

The following table is the ABB.163-derived Utility occurrence spectrum generated with the Appendix A.8 transformation. In controlled machine-readable form, it is the operational numerical reference spectrum for EFI. The full reference table, including ABB.163 printed labels and class edges, is part of the calculation package.

| q | LF center (g) | Occ. | q | LF center (g) | Occ. |
| --- | --- | --- | --- | --- | --- |
| 1 | -3.828 | 0 | 17 | 1.672 | 781,639 |
| 2 | -3.484 | 0 | 18 | 2.016 | 222,911 |
| 3 | -3.141 | 0 | 19 | 2.359 | 115,715 |
| 4 | -2.797 | 2 | 20 | 2.703 | 64,698 |
| 5 | -2.453 | 17 | 21 | 3.047 | 46,996 |
| 6 | -2.109 | 116 | 22 | 3.391 | 31,526 |
| 7 | -1.766 | 404 | 23 | 3.734 | 17,932 |
| 8 | -1.422 | 2,241 | 24 | 4.078 | 10,741 |
| 9 | -1.078 | 15,111 | 25 | 4.422 | 4,965 |
| 10 | -0.734 | 258,197 | 26 | 4.766 | 1,411 |
| 11 | -0.391 | 1,229,659 | 27 | 5.109 | 451 |
| 12 | -0.047 | 776,762 | 28 | 5.453 | 107 |
| 13 | 0.297 | 880,781 | 29 | 5.797 | 28 |
| 14 | 0.641 | 7,638,649 | 30 | 6.141 | 9 |
| 15 | 0.984 | 7,615,484 | 31 | 6.484 | 0 |
| 16 | 1.328 | 5,018,617 | 32 | 6.828 | 0 |

## C.4 Validation against ABB.164 by Kossira & Reinke (K&R) class

The generated ABB.163 spectrum was compared with the ABB.164 Utility occurrence curve digitized from the published graph. The comparison is performed by Kossira & Reinke (K&R) class index, not by using the imperfect digitized load-factor coordinates from the scanned figure.

The comparison statistics are:

Compared points:                      27
Median absolute relative difference:   2.7 %
Mean absolute relative difference:     2.9 %
Maximum absolute relative difference:  10.0 %
Points within ±5 %:                   25 / 27
Points within ±10 %:                  27 / 27

The maximum relative difference occurs at a low occurrence count where the absolute difference is one occurrence. Excluding this small-count point, the maximum relative difference is approximately 6%.

## C.5 Conclusion

The agreement between the ABB.163-derived occurrence spectrum and the ABB.164 published Utility curve confirms the Markov-matrix-to-occurrence-spectrum transformation and the Kossira & Reinke class convention used by this methodology. The SLM numerical reference spectrum is therefore the controlled ABB.163-derived occurrence table.

# 10. References

[1] Kossira, H. and Reinke, W. (OSTIV Publication XVI). “Determination of load spectra for the design of sailplanes.”

[2] Kossira, H. (1982). “Determination of load spectra and their application for keeping the operational life proof of sporting airplanes.” ICAS-Proc. 8/1982; ICAS-82-2.8.2, pp. 1330–1338.

[3] EASA Certification Memorandum CM-S-006, Certification, Type Design Definition, Material and Process Qualification for Composite Light Aircraft.

[4] Miner, M.A. (1945). Cumulative Damage in Fatigue. Journal of the Engineering Mechanics Division.

[5] Palmgren, A. (1947). A Probabilistic Theory of Cumulative Damage.

[6] Basquin, O.H. (1910). “The exponential law of endurance tests.” Proceedings of the ASTM, Vol. 10, pp. 625–630.

[7] Derouineau, J.L. (OSTIV Publication). “Extension of Glider Life using Structural Life Monitoring.”

[8] Kossira, H. and Reinke, W. “Festigkeit von modernen GFK-Konstruktionen für Segelflugzeuge - Bestimmung eines Belastungskollektives.” IFL-IB 84-01, Technische Universität Braunschweig.

[9] Mandell, J. F., Samborsky, D. D., Agastra, P., Sears, A. T. and Wilson, T. J. (2010). Analysis of SNL/MSU/DOE Fatigue Database Trends for Wind Turbine Blade Materials. Sandia Contractor Report SAND2010-7052. Sandia National Laboratories.

[10] Samborsky, D. D., Agastra, P. and Mandell, J. F. (2012). “The SNL/MSU/DOE Fatigue of Composite Materials Database: Recent Trends.” AIAA/ASME Wind Energy Symposium.

[11] Harris, B. (ed.) (2003). Fatigue in Composites: Science and Technology of the Fatigue Response of Fibre-Reinforced Plastics. Woodhead Publishing.

[12] Talreja, R. (1985). Fatigue of Composite Materials. Technomic Publishing.

[13] Bathias, C. (1999). Fatigue of Materials and Structures: Application to Design and Damage. Wiley.